| 1. Gather appropriate equipment. |  |  |  |
| --- | --- | --- | --- |
| 2. Properly greet and identify patient. Explain procedure. |  |  |  |
| 3. Choose a suitable puncture site and position patient adequately. |  |  |  |
| 4. Wash hands. Put on gloves. |  |  |  |
| - Open 6" extension tubing and remove from package. Prime tubing with saline and place connector end back in package. (Leave syringe attached to cap) |  |  |  |
| 6. Apply tourniquet properly. |  |  |  |
| 7. Prepare insertion site with iodine swabstick provided in start kit and allow to dry. |  |  |  |
| 8. Perform venipuncture successfully with minimum trauma to patient. |  |  |  |
| a. Hold needle with bevel up. |  |  |  |
| b. Use thumb of opposite hand to hold skin taut above the puncture site. |  |  |  |
| c. Push the needle through the skin at a 45 degree angle. |  |  |  |
| d. Lower the needle to 15-20 degrees and slowly advance to seat needle in the vein. |  |  |  |
| e. Remove the tourniquet. |  |  |  |
| - Holding the needle still, float the catheter off the needle and into the vein until the hub rests at the skin surface. Dispose of needle in sharps container. |  |  |  |
| 10. Attach the extension tubing to the catheter. |  |  |  |
| 11. Place tegaderm over the insertion site. |  |  |  |
| 12. Flush tubing and catheter with saline in syringe. (Use a minimum of 3cc of saline) |  |  |  |
| - 13. Date, initial and state catheter gauge on label supplied on the Tegaderm dressing. Apply to side of dressing. |  |  |  |
| 14. Secure extension to assure stability. |  |  |  |
| 15. Make patient comfortable. |  |  |  |
| 16. Document on IV record: start or restart of therapy, site, date and time started. |  |  |  |
| # INSTRUCTOR SIGNATURES. |  |  |  |

| NAME __________________________ | TITLE _______________________ |
| --- | --- |

CHECK IF SKILL PERFORMED

|  | DATE | DATE | DATE |
| --- | --- | --- | --- |
| ## PROFICIENCY EVALUATION VENIPUNCTURE |  |  |  |

SATISFACTORILY